Task 5: Capture and Analyze Network Traffic Using Wireshark

Objective: Capture live network packets and identify basic protocols and traffic types.

Tools: Wireshark (free).

1.  Captured packets using Wireshark

2. Ping to my Local campus Server

3. Filter captured packets by protocol (e.g., HTTP, DNS, TCP)

-http

-DNS

-TCP

4. Other Protocols

-ICMP

-SMB (Server Message Block)

-UDP

5. Summary of Findings (from your capture file)

During packet capture and analysis with Wireshark, the following protocols were identified:

- DNS (Domain Name System):

  - Found several DNS queries and responses.

  - Example: My system queried a DNS server to resolve a domain name (e.g., google.com) into an IP address.

  - Transport: UDP packets on port 53.

- TCP (Transmission Control Protocol):

  - Observed TCP 3-way handshake (SYN, SYN-ACK, ACK) before data transfer.

  - TCP segments carried higher-level protocols like HTTP and SMB.

  - Ports observed: 80 (HTTP), 445 (SMB).

- HTTP (Hypertext Transfer Protocol):

  - Captured HTTP GET and response packets when browsing a website.

  - Example: A request to fetch a webpage was visible.

- UDP (User Datagram Protocol):

  - Used for lightweight, connectionless communication.

  - Found in DNS queries and other background services.

- SMB (Server Message Block):

  - Detected file-sharing traffic on port 445.

  - Indicates local or background Windows network services.

- ICMP (Internet Control Message Protocol):

  - Observed echo request and echo reply packets (ping).

  - Example: My system sent ICMP requests to test connectivity and got replies.

6. Packet Details (Examples from Capture)

- DNS: Query from client → server 8.8.8.8, response with IP address.

- TCP Handshake: Source port 51520 → Destination port 80, packets with flags SYN, SYN-ACK, ACK.

- HTTP: GET /index.html request observed, followed by server response.

- ICMP: Echo request (Type 8) and echo reply (Type 0) between my system and external host.

- SMB: Traffic on TCP port 445 showing protocol negotiation packets.